# Revit Element Properties Manual

Application Guide for MC_PROJETOS and ECTools

This manual identifies the information that must be reviewed in Revit before a type is approved in MC_PROJETOS, where each value is edited, and which approved information ECTools can return to the model. It includes examples for every discipline and category currently supported by the application.

| Control | Value |
| --- | --- |
| Document | Operating manual for Revit type properties and data exchange |
| Application | MC_PROJETOS 04.5 Element Types |
| Exchange | ECTools JSON contract 2.0 |
| Status | Baseline guide to be adapted to project information requirements |
| Date | September 27, 2026 |

# What This Manual Controls

Operating rule. The BIM Manager defines and approves the standard in MC_PROJETOS. ECTools preserves the Revit type identity, validates each proposed change, and applies only APPROVED records. Model authors must not rename approved types independently.

ISO 19650 establishes the information management process. The project appointment and Information Standard define the required properties, classifications, and values.

A Type Name describes a type. Level, room, circuit, position, instance mark, and coordinates belong to an instance and must not be included in the Type Name.

MasterFormat, UniFormat, and OmniClass codes must come from the project-approved tables. The examples in this manual explain the field and its purpose; they do not replace the contractual classification library.

Confirm every IFC class against the exchange IFC version and model view definition. A class available in IFC 4.3 might not be available in an IFC 2x3 export.

# Editing and Approval Workflow

| Step | Tool | Action | Result |
| --- | --- | --- | --- |
| 1 | Revit and ECTools | Export the catalog by discipline. ECTools provides source_document_guid, record_id, revit_unique_id, category, family, type, and parameters. | FROM_REVIT batch with no deletion authority based on omission. |
| 2 | MC_PROJETOS | Filter by model, discipline, and category. Open Standards and complete naming, classification, IFC, and modeling strategy data. | Reviewed record; Revit has not been modified yet. |
| 3 | MC_PROJETOS | Change the status to APPROVED and export the MC_TO_REVIT package. | VALIDATE, RENAME, UPDATE, or CREATE action with stable keys. |
| 4 | ECTools | Run Validate and Preview before Apply. Do not apply PENDING, invalid, or unapproved records. | Revit is updated through a controlled operation. |
| 5 | Revit and MC_PROJETOS | Export and import MC_Revit_Result. | APPLIED, FAILED, SKIPPED, or PENDING traceability. |

# Revit Properties Interface Map

Recommended path. Select an element, click Edit Type, and work in Type Properties. Use Duplicate before CREATE or whenever a change must not affect every existing instance.

| Location in Revit | Property | Scope | Authority and Rule |
| --- | --- | --- | --- |
| Type selector | Type Name | Type | MC_PROJETOS generates the canonical name. ECTools renames or duplicates the type. Do not include LOD, complete layer build-ups, levels, or manufacturers unless the family is supplier-specific. |
| Identity Data | Type Mark | Type | Required and unique within the project for tags and schedules. |
| Identity Data | Description | Type | Readable functional description without unnecessary abbreviations. |
| Identity Data | Keynote | Type | MasterFormat code selected from the project-approved library. |
| Identity Data | Assembly Code | Type | UniFormat code used for system classification and early cost planning. |
| Identity Data or shared parameter | ClassificationCode | Type | OmniClass or another contractual classification code. Preserve the system and edition in the JSON contract. |
| IFC Parameters | IFC Export As and IFC Predefined Type | Type | IFC class and predefined type approved for the exchange. |
| Identity Data | Manufacturer | Type | Leave blank for generic types. Complete it only after selecting a supplier-specific product or family. |
| MC shared parameters | MC Modeling Strategy and MC Material Data Method | Type | Document the decomposition strategy and how material or subcomponent data is managed. |
| Properties palette | Mark, Level, Offset, Room, Circuit, Flow | Instance | Values that vary by occurrence. They are not part of the Type Name. |

# Required Fields Before Approval

| Field in MC_PROJETOS | Destination in Revit or JSON | Validation |
| --- | --- | --- |
| Discipline, category, function, system, variant, dimension | Contract fields and generated type_code | Clean code consistent with the appointed discipline. |
| Approved MC type name | Revit Type Name | Pattern DISCIPLINE_CATEGORY_FUNCTION_DIMENSION_DESCRIPTION with no LOD or complete layer build-up. |
| Function or Use | Function / Use | Functional value consistent with the category and system. |
| Type Mark | ALL_MODEL_TYPE_MARK | Required and unique. |
| Keynote | KEYNOTE_PARAM | Required project-approved MasterFormat code. |
| Assembly Code | UNIFORMAT_CODE | Required project-approved UniFormat code. |
| ClassificationCode | Shared parameter or mapped project parameter | Required. Preserve the classification system and edition. |
| Description | ALL_MODEL_DESCRIPTION | Complete functional description. |
| IFC entity and predefined type | IFC Export As / IFC Predefined Type | Required. Validate against the exchange schema. |
| Modeling strategy and material data method | MC shared parameters | Required. Separate elements by discipline, trade package, or construction phase when applicable. |

# ARC Architecture

Authoring elements that define spaces, envelope, access, finishes and interior equipment.

Common location. In 04.5 Element Types, open Standards; complete A Type name, B Revit parameters and classification, and C Modeling review. Export only APPROVED records.

| Revit category and code | Approved MC Type Name example | Type Mark and use | IFC mapping example | What to verify in Type Properties |
| --- | --- | --- | --- | --- |
| Walls WAL | ARC_WAL_EXT_250_Exposed brick facade 250 mm | W-01 EXT | IfcWall / STANDARD | Structure and total width; Function; Fire Rating; thermal/acoustic data; Structural flag where applicable. |
| Doors DOR | ARC_DOR_INT_0900X2100_Internal hinged door 900 x 2100 mm | D-02 INT | IfcDoor / DOOR | Width and height; operation; panel/frame material; Fire Rating; accessibility; From Room and To Room at instance level. |
| Windows WIN | ARC_WIN_EXT_1200X1200_Aluminium sliding window 1200 x 1200 mm | WN-03 EXT | IfcWindow / WINDOW | Width and height; glazing; frame material; thermal transmittance; acoustic/fire data; sill height at instance level. |
| Floors FLR | ARC_FLR_INT_100_Porcelain finished floor 100 mm | F-04 INT | IfcSlab / FLOOR | Compound structure and total thickness; Function; structural flag; finish and slip-resistance data through materials. |
| Roofs ROF | ARC_ROF_EXT_150_Inverted roof 150 mm | R-05 EXT | IfcRoof / NOTDEFINED | Compound structure; total thickness; Function; slope method; thermal and waterproofing data through materials. |
| Ceilings CLG | ARC_CLG_INT_120_Gypsum board suspended ceiling 120 mm | C-06 INT | IfcCovering / CEILING | Compound structure; thickness; acoustic/fire performance; height above level at instance level. |
| Furniture FUR | ARC_FUR_INT_01_Loose chair type 01 | FU-07 INT | IfcFurniture / NOTDEFINED | Overall dimensions; host/level behavior; accessibility clearance; manufacturer only for a supplier-specific family. |

Values That Must Not Be Included in the Type Name

Level, Room, From Room, To Room, sill height, individual Mark and phase.

# STR Structural concrete

Cast-in-place structural framing and slabs.

Common location. In 04.5 Element Types, open Standards; complete A Type name, B Revit parameters and classification, and C Modeling review. Export only APPROVED records.

| Revit category and code | Approved MC Type Name example | Type Mark and use | IFC mapping example | What to verify in Type Properties |
| --- | --- | --- | --- | --- |
| Structural Columns COL | STR_COL_STR_300X300_Reinforced concrete column 300 x 300 mm | C-01 STR | IfcColumn / COLUMN | Section dimensions/profile; material; structural material; analytical model; load-bearing role. |
| Structural Framing BEA | STR_BEA_STR_250X500_Reinforced concrete beam 250 x 500 mm | B-02 STR | IfcBeam / BEAM | Section/profile; material; structural usage; analytical model; camber or fabrication data when required. |
| Floors SLB | STR_SLB_STR_150_Reinforced concrete slab 150 mm | S-03 STR | IfcSlab / FLOOR | Thickness and structure; structural material; analytical model; span direction; openings modeled separately. |

Values That Must Not Be Included in the Type Name

Base/top level, offsets, analytical releases, individual Mark and construction sequence.

# STL Structural steel

Steel framing, columns and bracing.

Common location. In 04.5 Element Types, open Standards; complete A Type name, B Revit parameters and classification, and C Modeling review. Export only APPROVED records.

| Revit category and code | Approved MC Type Name example | Type Mark and use | IFC mapping example | What to verify in Type Properties |
| --- | --- | --- | --- | --- |
| Structural Columns COL | STL_COL_STR_HEB240_Steel column HEB 240 | C-01 STR | IfcColumn / COLUMN | Section dimensions/profile; material; structural material; analytical model; load-bearing role. |
| Structural Framing BEA | STL_BEA_STR_IPE300_Steel beam IPE 300 | B-02 STR | IfcBeam / BEAM | Section/profile; material; structural usage; analytical model; camber or fabrication data when required. |
| Structural Framing BRC | STL_BRC_STR_20_Steel bracing 20 mm | BR-03 STR | IfcMember / BRACE | Profile/diameter; material; structural usage; analytical model; connection data only at the required information need. |

Values That Must Not Be Included in the Type Name

Level, offsets, end joins, connection instance, piece mark unless fabrication workflow requires a separate controlled field.

# MAS Structural masonry

Load-bearing and non-load-bearing masonry coordinated with architecture.

Common location. In 04.5 Element Types, open Standards; complete A Type name, B Revit parameters and classification, and C Modeling review. Export only APPROVED records.

| Revit category and code | Approved MC Type Name example | Type Mark and use | IFC mapping example | What to verify in Type Properties |
| --- | --- | --- | --- | --- |
| Walls WAL | MAS_WAL_EXT_140_Concrete block wall 140 mm | W-01 EXT | IfcWall / STANDARD | Structure and total width; Function; Fire Rating; thermal/acoustic data; Structural flag where applicable. |
| Walls WAL | MAS_WAL_INT_90_Concrete block partition 90 mm | W-02 INT | IfcWall / STANDARD | Structure and total width; Function; Fire Rating; thermal/acoustic data; Structural flag where applicable. |

Values That Must Not Be Included in the Type Name

Level, base/top constraints, length, openings and phase.

# FDN Foundations

Shallow and deep foundations linked to the structural analytical model.

Common location. In 04.5 Element Types, open Standards; complete A Type name, B Revit parameters and classification, and C Modeling review. Export only APPROVED records.

| Revit category and code | Approved MC Type Name example | Type Mark and use | IFC mapping example | What to verify in Type Properties |
| --- | --- | --- | --- | --- |
| Structural Foundations FTG | FDN_FTG_STR_1200_Isolated footing 1200 mm | FT-01 STR | IfcFooting / NOTDEFINED | Length, width, thickness; structural material; analytical support; host column relationship. |
| Structural Foundations PIL | FDN_PIL_STR_600_Bored pile 600 mm | P-02 STR | IfcPile / NOTDEFINED | Diameter/section; length; material; cut-off level and founding level at instance level. |

Values That Must Not Be Included in the Type Name

Level, elevation, host, founding depth and individual Mark.

# PRE Precast

Prefabricated concrete elements whose fabrication identity must remain separate from cast-in-place work.

Common location. In 04.5 Element Types, open Standards; complete A Type name, B Revit parameters and classification, and C Modeling review. Export only APPROVED records.

| Revit category and code | Approved MC Type Name example | Type Mark and use | IFC mapping example | What to verify in Type Properties |
| --- | --- | --- | --- | --- |
| Walls or Structural Framing PNL | PRE_PNL_STR_200_Precast concrete panel 200 mm | PN-01 STR | IfcWall / STANDARD | Panel thickness and dimensions; structural material; connection/erection data kept in dedicated parameters. |
| Structural Framing BEA | PRE_BEA_STR_300X600_Precast concrete beam 300 x 600 mm | B-02 STR | IfcBeam / BEAM | Section/profile; material; structural usage; analytical model; camber or fabrication data when required. |
| Stairs STR | PRE_STR_STR_01_Precast concrete stair type 01 | ST-03 STR | IfcStair / PREFABRICATED | Width; rise; tread; landing geometry; material; precast piece identity when fabrication is required. |

Values That Must Not Be Included in the Type Name

Level, location, assembly instance, erection sequence and piece tracking fields.

# HID Hydraulic and sanitary

Water distribution, sanitary fixtures and valves.

Common location. In 04.5 Element Types, open Standards; complete A Type name, B Revit parameters and classification, and C Modeling review. Export only APPROVED records.

| Revit category and code | Approved MC Type Name example | Type Mark and use | IFC mapping example | What to verify in Type Properties |
| --- | --- | --- | --- | --- |
| Pipes PIP | HID_PIP_WAT_25_Cold water pipe PPR 25 mm | P-01 WAT | IfcPipeSegment / NOTDEFINED | Nominal diameter; material/specification; system classification; pressure/flow design data; insulation as separate data or element per project rule. |
| Plumbing Fixtures FIX | HID_FIX_SAN_STD_Water closet standard | PF-02 SAN | IfcSanitaryTerminal / NOTDEFINED | Connector systems; flow units; mounting; accessibility; manufacturer only for a selected product. |
| Pipe Accessories VLV | HID_VLV_WAT_25_Ball valve 25 mm | V-03 WAT | IfcValve / NOTDEFINED | Nominal diameter; pressure class; connection type; system classification; normally-open/closed state at instance level if required. |

Values That Must Not Be Included in the Type Name

Level, offset, flow, pressure result, system name and individual Mark.

# DRA Storm and sanitary drainage

Gravity drainage networks and chambers.

Common location. In 04.5 Element Types, open Standards; complete A Type name, B Revit parameters and classification, and C Modeling review. Export only APPROVED records.

| Revit category and code | Approved MC Type Name example | Type Mark and use | IFC mapping example | What to verify in Type Properties |
| --- | --- | --- | --- | --- |
| Pipes PIP | DRA_PIP_DRN_100_Drainage pipe PVC 100 mm | P-01 DRN | IfcPipeSegment / NOTDEFINED | Nominal diameter; material/specification; system classification; pressure/flow design data; insulation as separate data or element per project rule. |
| Generic Models PIT | DRA_PIT_DRN_600X600_Inspection chamber 600 x 600 mm | CH-02 DRN | IfcDistributionChamberElement / INSPECTIONCHAMBER | Internal dimensions; cover level and invert level at instance level; material; connected system. |

Values That Must Not Be Included in the Type Name

Invert elevation, slope, length, connected system and chamber cover level.

# MEC Mechanical HVAC

Air distribution and mechanical equipment.

Common location. In 04.5 Element Types, open Standards; complete A Type name, B Revit parameters and classification, and C Modeling review. Export only APPROVED records.

| Revit category and code | Approved MC Type Name example | Type Mark and use | IFC mapping example | What to verify in Type Properties |
| --- | --- | --- | --- | --- |
| Ducts DCT | MEC_DCT_AIR_400X200_Rectangular duct 400 x 200 mm | DC-01 AIR | IfcDuctSegment / NOTDEFINED | Width/height or diameter; material; system classification; pressure class; lining and insulation according to the decomposition rule. |
| Mechanical Equipment EQP | MEC_EQP_AIR_01_Air handling unit type 01 | EQ-02 AIR | IfcUnitaryEquipment / NOTDEFINED | Capacity; airflow/power; voltage where applicable; connectors; maintenance clearance; manufacturer only for selected equipment. |
| Air Terminals DIF | MEC_DIF_AIR_600X600_Supply diffuser 600 x 600 mm | AT-03 AIR | IfcAirTerminal / NOTDEFINED | Face/neck dimensions; airflow; throw; system classification; mounting height at instance level. |

Values That Must Not Be Included in the Type Name

Level, offset, airflow result, system name, circuit/controls and individual Mark.

# ELE Electrical power and lighting

Power distribution, panels and lighting devices.

Common location. In 04.5 Element Types, open Standards; complete A Type name, B Revit parameters and classification, and C Modeling review. Export only APPROVED records.

| Revit category and code | Approved MC Type Name example | Type Mark and use | IFC mapping example | What to verify in Type Properties |
| --- | --- | --- | --- | --- |
| Wires or Cable Trays CAB | ELE_CAB_PWR_3X25_Copper power cable 3 x 25 | CB-01 PWR | IfcCableSegment / NOTDEFINED | Cable size/conductors; material; voltage/service; circuit relationship at instance level; do not encode circuit number in the type name. |
| Electrical Equipment PNL | ELE_PNL_PWR_01_Distribution board type 01 | PN-02 PWR | IfcElectricDistributionBoard / NOTDEFINED | Rated capacity; voltage; poles/phases; enclosure; distribution system; panel name remains an instance identifier. |
| Lighting Fixtures LGT | ELE_LGT_LGT_600X600_LED luminaire 600 x 600 mm | L-03 LGT | IfcLightFixture / NOTDEFINED | Power; luminous flux; color temperature; emergency function; circuit and switching at instance level. |

Values That Must Not Be Included in the Type Name

Level, circuit number, panel assignment, load result and individual Mark.

# FIR Fire fighting

Fire suppression pipework and terminals.

Common location. In 04.5 Element Types, open Standards; complete A Type name, B Revit parameters and classification, and C Modeling review. Export only APPROVED records.

| Revit category and code | Approved MC Type Name example | Type Mark and use | IFC mapping example | What to verify in Type Properties |
| --- | --- | --- | --- | --- |
| Pipes PIP | FIR_PIP_FIR_100_Fire protection pipe 100 mm | P-01 FIR | IfcPipeSegment / NOTDEFINED | Nominal diameter; material/specification; system classification; pressure/flow design data; insulation as separate data or element per project rule. |
| Sprinklers SPR | FIR_SPR_FIR_STD_Pendent sprinkler standard | SP-02 FIR | IfcFireSuppressionTerminal / SPRINKLER | K factor; response; temperature rating; orientation; system classification; elevation at instance level. |

Values That Must Not Be Included in the Type Name

Level, elevation, system name and individual Mark.

# ELV Telecommunications and data

Low-voltage communication cabling and outlets.

Common location. In 04.5 Element Types, open Standards; complete A Type name, B Revit parameters and classification, and C Modeling review. Export only APPROVED records.

| Revit category and code | Approved MC Type Name example | Type Mark and use | IFC mapping example | What to verify in Type Properties |
| --- | --- | --- | --- | --- |
| Wires or Cable Trays CAB | ELV_CAB_DAT_CAT6_Data cable CAT6 | CB-01 DAT | IfcCableSegment / NOTDEFINED | Cable size/conductors; material; voltage/service; circuit relationship at instance level; do not encode circuit number in the type name. |
| Data Devices OUT | ELV_OUT_DAT_RJ45_Data outlet RJ45 | O-02 DAT | IfcOutlet / DATAOUTLET | Connector/service; ports; mounting type; voltage/data standard; room and circuit at instance level. |

Values That Must Not Be Included in the Type Name

Level, circuit, rack/patch assignment, room and individual Mark.

# GAS Gas

Fuel-gas distribution and isolation devices.

Common location. In 04.5 Element Types, open Standards; complete A Type name, B Revit parameters and classification, and C Modeling review. Export only APPROVED records.

| Revit category and code | Approved MC Type Name example | Type Mark and use | IFC mapping example | What to verify in Type Properties |
| --- | --- | --- | --- | --- |
| Pipes PIP | GAS_PIP_GAS_25_Gas pipe steel 25 mm | P-01 GAS | IfcPipeSegment / NOTDEFINED | Nominal diameter; material/specification; system classification; pressure/flow design data; insulation as separate data or element per project rule. |
| Pipe Accessories VLV | GAS_VLV_GAS_25_Gas ball valve 25 mm | V-02 GAS | IfcValve / NOTDEFINED | Nominal diameter; pressure class; connection type; system classification; normally-open/closed state at instance level if required. |

Values That Must Not Be Included in the Type Name

Level, offset, flow/pressure result, system name and individual Mark.

# SYS General building systems

Temporary or project-specific system equipment pending assignment to a specialist discipline.

Common location. In 04.5 Element Types, open Standards; complete A Type name, B Revit parameters and classification, and C Modeling review. Export only APPROVED records.

| Revit category and code | Approved MC Type Name example | Type Mark and use | IFC mapping example | What to verify in Type Properties |
| --- | --- | --- | --- | --- |
| Mechanical Equipment EQP | SYS_EQP_MEP_01_Building system equipment type 01 | EQ-01 MEP | IfcUnitaryEquipment / NOTDEFINED | Capacity; airflow/power; voltage where applicable; connectors; maintenance clearance; manufacturer only for selected equipment. |

Values That Must Not Be Included in the Type Name

Level, room, system assignment, calculated results and individual Mark.

# CIV Civil and earthworks

External paving and engineered ground works.

Common location. In 04.5 Element Types, open Standards; complete A Type name, B Revit parameters and classification, and C Modeling review. Export only APPROVED records.

| Revit category and code | Approved MC Type Name example | Type Mark and use | IFC mapping example | What to verify in Type Properties |
| --- | --- | --- | --- | --- |
| Toposolids or Floors PAV | CIV_PAV_EXT_80_Asphalt pavement 80 mm | PV-01 EXT | IfcPavement / NOTDEFINED | Layer structure and thickness; material; function; slope/elevation at instance level; separate pavement from subgrade when contracts/phases differ. |
| Toposolids GRD | CIV_GRD_EXT_01_Engineered fill type 01 | G-02 EXT | IfcEarthworksFill / NOTDEFINED | Material class; compaction requirement; phase; design elevation and volume reporting. |

Values That Must Not Be Included in the Type Name

Elevation, slope, phase, quantity result and location identifier.

# ROA Roads and pavement

Road surfaces and pavement assemblies.

Common location. In 04.5 Element Types, open Standards; complete A Type name, B Revit parameters and classification, and C Modeling review. Export only APPROVED records.

| Revit category and code | Approved MC Type Name example | Type Mark and use | IFC mapping example | What to verify in Type Properties |
| --- | --- | --- | --- | --- |
| Toposolids or Floors PAV | ROA_PAV_EXT_100_Road asphalt pavement 100 mm | PV-01 EXT | IfcPavement / NOTDEFINED | Layer structure and thickness; material; function; slope/elevation at instance level; separate pavement from subgrade when contracts/phases differ. |

Values That Must Not Be Included in the Type Name

Station, elevation, slope, phase and segment identifier.

# ACO Acoustics

Acoustic treatments coordinated with architectural hosts.

Common location. In 04.5 Element Types, open Standards; complete A Type name, B Revit parameters and classification, and C Modeling review. Export only APPROVED records.

| Revit category and code | Approved MC Type Name example | Type Mark and use | IFC mapping example | What to verify in Type Properties |
| --- | --- | --- | --- | --- |
| Ceilings or Specialty Equipment PNL | ACO_PNL_INT_50_Acoustic panel 50 mm | PN-01 INT | IfcCovering / NOTDEFINED | Panel dimensions; acoustic performance; finish and host requirements kept in dedicated parameters. |

Values That Must Not Be Included in the Type Name

Host, room, level, area and installation phase.

# SUS Sustainability

Renewable-energy devices and performance-related objects.

Common location. In 04.5 Element Types, open Standards; complete A Type name, B Revit parameters and classification, and C Modeling review. Export only APPROVED records.

| Revit category and code | Approved MC Type Name example | Type Mark and use | IFC mapping example | What to verify in Type Properties |
| --- | --- | --- | --- | --- |
| Electrical Equipment DEV | SUS_DEV_ENV_01_Photovoltaic panel type 01 | PV-01 ENV | IfcSolarDevice / SOLARPANEL | Rated capacity; efficiency; electrical connectors; orientation/tilt at instance level; supplier data only after selection. |

Values That Must Not Be Included in the Type Name

Level, orientation, circuit, calculated generation and asset serial number.

# TOP Topography and survey

Survey control information tied to the approved coordinate reference system.

Common location. In 04.5 Element Types, open Standards; complete A Type name, B Revit parameters and classification, and C Modeling review. Export only APPROVED records.

| Revit category and code | Approved MC Type Name example | Type Mark and use | IFC mapping example | What to verify in Type Properties |
| --- | --- | --- | --- | --- |
| Generic Models or Site PNT | TOP_PNT_SUR_01_Survey control point type 01 | CP-01 SUR | IfcAnnotation / NOTDEFINED | Point purpose; coordinate reference; accuracy class; coordinates are controlled instance data and must not be encoded in the type name. |

Values That Must Not Be Included in the Type Name

Northing, easting, elevation, point number and survey observation date.

# GEO Geotechnical

Ground investigation locations and linked factual records.

Common location. In 04.5 Element Types, open Standards; complete A Type name, B Revit parameters and classification, and C Modeling review. Export only APPROVED records.

| Revit category and code | Approved MC Type Name example | Type Mark and use | IFC mapping example | What to verify in Type Properties |
| --- | --- | --- | --- | --- |
| Generic Models or Site BHR | GEO_BHR_SUR_01_Borehole record type 01 | BH-01 SUR | IfcBorehole / NOTDEFINED | Investigation method; diameter; datum; depth at instance level; strata and test results in linked records or dedicated properties. |

Values That Must Not Be Included in the Type Name

Coordinates, ground level, termination depth, sampling records and test results.

# Closeout Checklist Before Returning Data to Revit

| No | Control | Question | Status |
| --- | --- | --- | --- |
| 1 | Identity | source_document_guid, record_id and revit_unique_id remain unchanged for existing types. | Required |
| 2 | Naming | Canonical Type Name contains discipline, category/location, function/subtype, main dimension/capacity and concise description. | Required |
| 3 | Prohibited content | No LOD, full layer build-up, level, room, circuit or manufacturer in the generic Type Name. | Required |
| 4 | US classification | Type Mark, MasterFormat Keynote, UniFormat Assembly Code and project classification code are populated. | Required |
| 5 | IFC | Entity and predefined type match the project export schema and MVD. | Required |
| 6 | Decomposition | Multiple trades/phases are separated or an approved modeling exception is documented. | Required |
| 7 | Action | VALIDATE, RENAME, UPDATE or CREATE is correct. CREATE includes base_family and base_type. | Required |
| 8 | Approval | status equals APPROVED only after review by the responsible project role. | Required |
| 9 | Preview | ECTools Validate and Preview show the intended changes before Apply. | Required |
| 10 | Result | MC_Revit_Result is returned to MC_PROJETOS and unresolved FAILED or PENDING records are corrected. | Required |

# Plugin data requirements

ECTools must preserve identity. For an existing type, source_document_guid, record_id, revit_unique_id, family_name, base_family and base_type remain intact. Type Name is editable but is not a primary key.

| Contract area | Fields or parameter records required |
| --- | --- |
| Element identity | source_document_guid, record_id, revit_unique_id, revit_element_id, family_name, type_name, base_family and base_type |
| Category | discipline, category_code, revit_category and revit_category_id |
| MC nomenclature | function_code, system_code, variant_code, dimension, type_code and readable_name |
| Classification | classification_system, classification_version, classification_code, Keynote and Assembly Code |
| IFC | ifc_entity and ifc_predefined_type |
| Standard parameters | parameters_set records for Type Mark, Description, Function / Use, Manufacturer, IFC Export As, IFC Predefined Type, MC Modeling Strategy, MC Material Data Method, MC Multiple Trades and MC Multiple Phases |
| Application control | action, status, batch_id, contract_sha256 and result state |

# How to use this manual in a real project

At kickoff, the BIM Manager confirms the contracted disciplines, classification editions, shared-parameter file, IFC schema and project-specific required properties.

Each discipline reviews only its categories in MC_PROJETOS using the model, discipline and category filters.

Category examples are copied and adapted; they are not universal product specifications.

The approved project template and family library remain the source for parameter GUIDs, units, connectors and geometric behavior.

The project Information Standard records approved deviations and supersedes generic examples in this manual.