BIM Execution Plan (BEP)

Information-management execution plan. Change-managed document — revisions recorded in Section A.

Project: {{PROJECT_NAME}}   ·   Code: {{CODE}}   ·   Client: {{CLIENT}}   ·   Address: {{ADDRESS}}

Author / Lead Appointed Party: {{AUTHOR}}   ·   Version {{DOC_VERSION}}   ·   Date: {{DOC_DATE}}   ·   Container: {{CODE}}-{{OR}}-XX-RP-0005_BEP_v1.0

Contents

Working protocol: MC applies the publicly verifiable direction of ISO/DIS 19650-1 and ISO/DIS 19650-2 (2026 drafts) to project information management. ISO 19650-1:2018 and ISO 19650-2:2018 remain the published references until replaced. The appointment and applicable national adoption determine contractual obligations. Generation does not constitute review, authorization, acceptance or certification.

A. Revision history & BEP change management

| Version | Date | Author | Change |
| --- | --- | --- | --- |
| 1.0 | {{DOC_DATE}} | {{AUTHOR}} | Initial issue at project kickoff |

Change-management process for this BEP: any Party may propose a change to the {{BIM_MANAGER}} (BIM Manager). The BIM Manager assesses impact (scope, roles, information production schedule), approves or rejects, and if approved re-issues the BEP with a new version and a new row in this history. Model splits (Section G) and role changes are always processed as BEP changes.

B. Project information

Project: {{PROJECT_NAME}}. Code: {{CODE}}. Client (Appointing Party): {{CLIENT}}. Address: {{ADDRESS}}. Lead Appointed Party: {{AUTHOR}}. Working basis: {{NORM_WORKING}}. Published/contractual reference: {{NORM_PUBLISHED}}.

C. BIM uses — included, excluded, future

C.1 Included

- Existing conditions / site model.

- Structural analysis (Revit ↔ Robot) and foundation verification (Midas GTS NX).

- Hydraulic modelling.

- 3D coordination and clash detection (Revit Interference Check).

- Quantity take-off and construction documentation.

C.2 Excluded

- Architecture authoring (external, AR — reference only).

- Geotechnical investigation and topographic survey (external, GE).

- Fire-fighting (FIR) and electrical (ELE) engineering.

- 4D scheduling, 5D cost automation, energy analysis, FM/asset data (unless later agreed).

C.3 Possible future uses

- Asset management data drop at handover (if the Client confirms AIR).

- 4D/5D and fire-fighting/electrical if scope is extended (processed as a BEP change).

D. Information delivery strategy

Information is produced in WIP, checked, and shared through the CDE at milestones M1–M5 per the information production schedule ({{CODE}}-{{OR}}-XX-SP-0010). Level of information need per the IE Worksheet (ISO 7817-1:2024). Milestone dates are [TO BE DEFINED] and held in the information production schedule — never invented.

E. Assignment matrix and ISO 19650 parties

| ISO 19650 party / function | Holder | Authority |
| --- | --- | --- |
| {{ROW_ROLE}} | {{ROW_PERSON}} | {{ROW_AUTHORITY}} |